| 5000 watts | Electric oven |
| --- | --- |
| 5000 watts | Clothes dryer (electric) |
| 3800 watts | Water heater (electric) |
| 3500 watts | Central Air Conditioner (2.5 tons) |
| 1500 watts | Microwave oven |
| 1500 watts | Toaster (four-slot) |
| 900 watts | Coffee maker |
| 800 watts | Range burner |
| 500-1440 watts | Window unit air conditioner |
| 200-700 watts | Refrigerator |
| 60-100 watts | Light bulb (energy hog because houses have lots of lights, and it's easy to leave them on when they're not being used) |
| Fans |  |
| 100 watts | Floor fan or box fan (high speed) |
| 15-95 watts | Ceiling fan (Bigger fans and faster speeds use more energy. My 2004 42" Hampton Bay uses 24/28/42 watts on low/med/high respectively, according to the manual. Progress Energy says on high speed fans use 55/75/95 watts for 36"/48"/52" models respectively.) |
| Computers (see more about electrical use of computers) |  |
| 140-330 watts | Desktop Computer & 17" CRT monitor |
| 1-20 watts | Desktop Computer & Monitor (in sleep mode) |
| 120 watts | 17" CRT monitor |
| 40 watts | 17" LCD monitor |
| 45 watts | Laptop computer |
| Other |  |
| 60-100 watts | Regular light bulb |
| 4-165 watts | Video game (While playing game, 30W for PS2, 70W for XBox, and 165W for XBox 360. See full report at DX Gaming) |
| 55-90 watts | 19" television |
| 18 watts | Compact fluorescent light bulb |
| 4 watts | Clock radio |
| 3 watt-hours | Total power stored by an alkaline AA battery. This is to put batteries into perspective. If you could power your clock radio with a AA battery, it wouldn't |